Aircraft Equipment Suffixes

| Suffix | Equipment Capability | Suffix | Equipment Capability |
| --- | --- | --- | --- |
|  | NO DME |  | ADVANCED RNAV WITH TRANSPONDER AND MODE C |
| /X | No transponder | /E | FMS with DME/DME and IRU position updating |
| /T | Transponder with no Mode C | /F | FMS with DME/DME position updating |
| /U | Transponder with Mode C | /G | Global Navigation Satellite System (GNSS), including GPS or WAAS, with enroute and terminal capability. |
|  | DME |  |  |
| /D | No transponder | /R | Required Navigational Performance. The aircraft meets the RNP type prescribed for the route segment(s), route(s) and/or area concerned. |
| /B | Transponder with no Mode C |  |  |
| /A | Transponder with Mode C |  |  |
|  | TACAN ONLY |  | Reduced Vertical Separation Minimum (RVSM). Prior to conducting RVSM operations within the U.S., the operator must obtain authorization from the FAA or from the responsible authority, as appropriate. |
| /M | No transponder |  |  |
| /N | Transponder with no Mode C |  |  |
| /P | Transponder with Mode C |  |  |
|  | AREA NAVIGATION (RNAV) | /J | /E with RVSM |
| /Y | LORAN, VOR/DME, or INS with no transponder | /K | /F with RVSM |
| /C | LORAN, VOR/DME, or INS, transponder with no Mode C | /L | /G with RVSM |
| /I | LORAN, VOR/DME, or INS, transponder with Mode C | /Q | /R with RVSM |
|  |  | /W | RVSM |